Network Infrastructure Design – Documentation (Phase 1 to Phase 3)

Phase 1 – Network Design & VLSM

This phase focused on designing the logical network topology and calculating IP addressing using VLSM. The base network 172.16.0.0/16 was subnetted to support the   following  departments with efficient address usage.

| Department | Hosts Required Hosts Required |
| --- | --- |
| Sales | 100 |
| Finance | 60 |
| IT Support | 30 |
| WiFi | 10 |
| Servers | 5 |
| Servers | 2 |

VLSM Calculation Steps:
• Networks were ordered from largest to smallest host requirement.
• Each subnet was allocated the minimum number of addresses required.
• Subnets were assigned sequentially to avoid overlap.

- Sales Departement -100 Hosts (largest).

Total Addresses needed:  100+2=102.

Required Power of 2: 102<=128

Host bits(H)=7 and prefix length(N)=32-7=25

New Subnet Mask: /25 in CIDR Notation

The block size =256-128=128 and the Decimal Mask will be 255.255.255.128

Address Block: Starting from the original network address.

Network Address: 172.16.0.0

Broadcast Address: 172.16.0.0  + 127 = 172.16.0.127

Usable IP Range: 172.16.0.1 to 172.16.0.126

Next Available Subnet: 172.16.0.128

VLAN and VLSM Allocation Table:

| VLAN | Department | Network/Subnet | Default Gateway |
| --- | --- | --- | --- |
| 10 | Sales | 172.16.0.0/25 | 172.16.0.1 |
| 20 | Finance | 172.16.0.128/26 | 172.16.0.129 |
| 30 | IT Support | 172.16.0.192/27 | 172.16.0.193 |
| 40 | WiFi | 172.16.0.224/28 | 172.16.0.225 |
| 50 | Servers | 172.16.0.240/29 | 172.16.0.241 |
| WAN | ISP Link | 172.16.0.248/30 | 172.16.0.249 / 172.16.0.250 |

Phase 2 – Switch Configuration

In this phase, VLANs were created on all switches and access ports were assigned based on departmental roles. Trunk links were configured between switches and the router to support inter-VLAN communication.

Key configurations included:

- VLAN creation (10, 20, 30, 40, 50)

- Access port assignment per department

- Trunk configuration between SW1–SW2, SW1–SW3, and SW1–Router

- Resolution of native VLAN mismatch issues by standardizing native VLAN to VLAN 1

Phase 3 – Router Configuration

Phase 3 implemented routing, WAN connectivity, and internet access simulation. Router-on-a-Stick was used for inter-VLAN routing, with additional routing and NAT features.

Router configurations included:

- Sub-interfaces on GigabitEthernet0/0 for each VLAN using 802.1Q encapsulation

- EIGRP (AS 100) for internal routing

- BGP peering with ISP router (AS 65002)

- NAT overload configured for internal networks

- Serial WAN link configuration to ISP

At the end of Phase 3, inter-VLAN routing, internal routing, and external connectivity were successfully verified, preparing the network for service deployment in Phase 4.